NR-2213 (NTSC) Color Camera

[image: image1.png]



· Bracket mount Type

· Size: Ø25 x 31 mm (L)
· Output: NTSC
· Diagonal Angle: 170° Diagonal

· Working Temperature: -10° ~ + 50°C
· 1/4" color CMOS camera
· with 2M power cable

· with 6M wire cable

· Waterproof IP68

· Good performance under low LUX

· Easy to install and combine with car display

· Compact size

[image: image2.jpg]



[image: image3.jpg]User Manual for Vehicle Vision System

Type:Bracket mounting series
Iltem No.: NR-2203
A - Installation Diagram

RADIO Panel

o Camera 1N
[ 1] o

|
E

. ~
iy A
(e ———
J Wire of back lamp

L:6M

B - Installation:
1.Looking for a suitable position on the trunk lid (Mark A) to install the camera.
2.Extend the Camera Cable(Mark C) through thelicense plate, along with the trunk lid and fix the cable line.
3.Find the power wire (B+) & (GND) wire of back lamp in the bounded power wires for back lamps.

4 Separately connect the (B+) & (GND) of Wire Cable with the (B+) & (GND) of back lamp wire and fix by tape on the
connector position. Plug in the connector (Mark C) into the Mark D and set the cable line on a suitable position.

5.Extend the wire cable through the side of back chair, along with the button line of the car doors to the side of the
Audio device.

6.Take apart the audio device (or monitor), plug in the connector (Mark E) into the hole of "Camera IN" which is
situated at the back of the audio device or monitor and reinstall the audio device.

7.Turn on the power for testing the image.
8.Adjust the position Mark upward and insert in into the drilled round hole (Mark A).

C ~ Troubleshooting:

1.The rear view camera couldn't be used when the car is in reverse and the display is belong to original equipment
(connect DVD and VCD), User should to adjust the key function of audio and search the image.

2.If the user don't want to search image via adjust the key function of audio, also could add the "signal auto switch
(Real View priority)" into this system, and the rear view camera will be used when the car is in reverse.

3.Please check the power of connect to the back lamp if the rear view camera couldn't be used and has eliminated
above point 1.

4.Please check the connector of Mark C and Mark D if the rear view camera couldn't be used and has eliminated
above point 1 and 3.

D - Note:

For safety reason, use only our cable line inside which some power-protective components are built, service is not
guaranteed if the product is damaged with other cable line being used




